Introduction to Digital Logic

EECS/CSE 31L

Final Project: Simple Processor

Prepared by: Jack Melcher

Student ID: 67574625

EECS Department

Henry Samueli School of Engineering

University of California, Irvine

March 10, 2015

1	Block Description

The processor fetches a 32-bit set of instructions, decodes the instructions in a controller, performs the specified task, and stores the results into a register file.

The processor is made up of several components.

- Counter (assignment 4)

- Memory

- Controller

- Register File (assignment 4)

- Selector for A input

- Selector for B input

- ALU 32 (assignment 3)

2	Input/Output Port Description

Package(c31L_pack): Contains all the necessary constants used throughout all the blocks

| Constant Name | Data Type | Default Value | Description |
| --- | --- | --- | --- |
| BW | INTEGER | 32 | Size of logic that is being used |
| OP_BINARY | STD_LOGIC | 010010 (18) | Number of Operations in the Memory |
| Reg_field | INTEGER | 6 | Determines size of register file and used to as its addr size |
| NBIT | INTEGER | 6 | Max bits the counter uses |
| Alu_function_type | std_logic_vector | 3 downto 0 | Used to help tie an alu operation to a given number |
| Mux_instructions | Array of std_logic_vector | ((OP-1) downto 0) (BW-1 downto 0) | Stores all the hard coded instructions for the processor |

Processor (The outputs are used to help display values of processor instruction results)

| Port name | Port size | Port Type | Description |
| --- | --- | --- | --- |
| clk | 1 | IN | Trigger the processor fetch and write events |
| Pc_enable | 1 | IN | Enable the counter to count |
| Rst_s | 1 | IN | Synchronous reset for the Counter and RegFile |
| memory_addrs | NBIT-1 downto 0 | OUT | Determine which instruction is read |
| Instruction | BW-1 downto 0 | OUT | Shows the 32 bit instruction |
| reg_source | BW-1 downto 0 | OUT | The first value read from register file |
| reg_target | BW-1 downto 0 | OUT | The second value read from register file |
| immediate | BW-1 downto 0 | OUT | The immediate value from instruction |
| carry | 1 | OUT | The carry out value from ALU |
| output | BW-1 downto 0 | OUT | Results of ALU operation |

Counter

| Port name | Port size | Port Type | Description |
| --- | --- | --- | --- |
| Clk | 1 | IN | Triggers the counting |
| Rst_s | 1 | IN | Will reset the counter to zero when active |
| dout | NBIT-1 downto 0 | OUT | The counter’s value after counting |

| Variable Name | Data Type | Port size | Description |
| --- | --- | --- | --- |
| count | INTEGER | NBIT-1 downto 0 | Aids in the counting process |

Memory

| Port name | Port size | Port Type | Description |
| --- | --- | --- | --- |
| addr | NBIT-1 downto 0 | IN | Determines which instructions are fetched |
| Read_data | BW-1 downto 0 | OUT | The instructions that are sent to controller |

Controller

| Port name | Port size | Port Type | Description |
| --- | --- | --- | --- |
| instruction | 31 downto 0 | IN | Instructions fetched from memory |
| rt_and_imm | 31 downto 0 | OUT | Immediate value |
| write_enable | 1 | OUT | Determines whether the register file can write |
| rs_index | 2 downto 0 | OUT | Address of register source |
| rt_index | 1 | OUT | Address of register target |
| rd_index | 1 | OUT | Address of register destination |
| b_mux_sel | 31 downto 0 | OUT | Instruction type to determine B input of ALU |
| alu_func | alu_function_type | OUT | Operation the ALU is supposed to perform |

Register File

| Port name | Port size | Port Type | Description |
| --- | --- | --- | --- |
| Clk | 1 | IN | Triggers the register process |
| Rst_s | 1 | IN | Will reset the registers to zero |
| write_enable | 1 | IN | Enables writing to the registers |
| rs_index | Reg_field -1 downto 0 | IN | The address of the register to be read from |
| rt_index | Reg_field -1 downto 0 | IN | The address of the register to be read from |
| rd_index | Reg_field -1 downto 0 | IN | The address of the register to be written to |
| reg_source_out | BW-1 downto 0 | OUT | The data value read from register file |
| reg_target_out | BW-1 downto 0 | OUT | The data value read from register file |
| reg_dest_new | BW-1 downto 0 | IN | The data value to be written to the register |

| Variable Name | Data Type | Port size | Description |
| --- | --- | --- | --- |
| regfile | Array (0 to 2** Reg_field) | NBIT-1 downto 0 | The registers |

Selector for A input

| Port name | Port size | Port Type | Description |
| --- | --- | --- | --- |
| reg_source | 31 downto 0 | IN | The data value read from register file |
| immediate | 31 downto 0 | IN | Immediate provided by the controller |
| alu_op | 1 | INOUT | The expected ALU operation to be performed |
| instruction_type |  |  | Selects A input based on RI value |
| opsel | 2 downto 0 | IN | The ALU operation select value |
| mode | 1 | OUT | The ALU mode select value |
| output | 31 downto 0 | OUT | What value is sent to A input of ALU |

Selector for B input

| Port name | Port size | Port Type | Description |
| --- | --- | --- | --- |
| in0 | BW-1 downto 0 | IN | Register target value |
| in1 | BW-1 downto 0 | IN | Immediate value |
| sel | 1 | INOUT | Selects B input based on RI value |
| output | BW-1 downto 0 |  | What value is sent to B input of ALU |

ALU 32-bit

| Port name | Port size | Port Type | Description |
| --- | --- | --- | --- |
| A | 31 downto 0 | IN | Value of first input |
| B | 31 downto 0 | IN | Value of second input |
| Cin | 1 | SIGNAL | The carry in value |
| opsel | 2 downto 0 | IN | Determine which operation to perform in units |
| mode | 1 | OUT | Determine whether the output is from arithmetic operation or logic operation |
| cout | 1 | OUT | The carry out value |
| output | 31 downto 0 | OUT | The output value of operation |

3	Design Schematics

Instruction Format

| 1 bit | 6 bit | 6 bit | 4 bit | 6 bit | 9bit |
| --- | --- | --- | --- | --- | --- |
| Instruction Type (RI) | Register Source (rs) | Register Destination (rd) | Function | Register Target (rt) | Immediate (Imm) |
| Whether ALU works with Immediate | Address of rs | The address for which the results of ALU will be written to | Code for ALU operation | Address of rt and the first 6 bits of Immediate | The last 9 bits of the Immediate |

ALU Operations:

| Function | Register Operation (RI = ‘0’) | Immediate Operation (RI = ‘1’) | Description |
| --- | --- | --- | --- |
| 0000 | NOP | NOP | No Operation |
| 0001 | rs + rt | a+ Imm | Add |
| 0010 | rs + rt ' | a+ Imm ' | Sub |
| 1011 | rs | Imm | Move |
| 1100 | rs +1 | Imm +1 | increment |
| 1101 | rs -1 | Imm -1 | decrement |
| 1110 | rs + rt +1 | a+ Imm +1 | Add & Increment |
| 0101 | rs AND rt | a AND Imm | Bit-wise AND |
| 0110 | rs OR rt | a OR Imm | Bit-wise OR |
| 1000 | rs XOR rt | a XOR Imm | Bit-wise Exclusive OR |
| 0111 | rs ' | Imm' | Compliment |
| 1001 | shl rs | shl Imm | 1 Bit Shift Left |

Design:

4	Compilation

Processor

C31L_pack

Counter

Memory

Controller

Register File

Selector for A input

Selector for B input

ALU 32

Testbench

5	Elaboration

Assumptions:

Functions

- Shifting left the register source by an immediate number of times was altered to just be shift left immediate value one bit

- Because the ALU could do increment, decrement, and add & increment, these functions were given their own code and can be used

Counter

- Need only to increment and then loop back to the first instruction

Memory

- The instructions had to be hard coded into this component

- Register 1 and register 2 in the Register File are assigned values using the MOVI funciton and then those values are used for the remainder of the

ALU

- This didn’t need any adjustments aside from the change of the cin port to a signal

A input

- Because some functions asked for the Immediate to be moved or shifted, another component was designed to handle this.

- In order to translate the function code to the ALU opsel and mode, this component also had to handle it.

Errors and Challenges:

- Lot of errors encountered when trying to make sure all the components are wired together in the correct sequence.

- A lot of assumptions had to be throughout the course of the final project

- It took a lot of time to design the schematic for the processor because of these assumptions

Simulation Log:

Started : "Simulate Behavioral Model".

Determining files marked for global include in the design...

Running fuse...

Command Line: fuse -intstyle ise -incremental -o {C:/Users/Jack/Desktop/EECS 31L/Final Project/FinalProject_67574625_cpu/testbench_processor_isim_beh.exe} -prj {C:/Users/Jack/Desktop/EECS 31L/Final Project/FinalProject_67574625_cpu/testbench_processor_beh.prj} work.testbench_processor {}

Running: C:\Xilinx\14.7\ISE_DS\ISE\bin\nt64\unwrapped\fuse.exe -intstyle ise -incremental -o C:/Users/Jack/Desktop/EECS 31L/Final Project/FinalProject_67574625_cpu/testbench_processor_isim_beh.exe -prj C:/Users/Jack/Desktop/EECS 31L/Final Project/FinalProject_67574625_cpu/testbench_processor_beh.prj work.testbench_processor

ISim P.20131013 (signature 0x7708f090)

Number of CPUs detected in this system: 4

Turning on mult-threading, number of parallel sub-compilation jobs: 8

Determining compilation order of HDL files

Parsing VHDL file "C:/Users/Jack/Desktop/EECS 31L/Final Project/FinalProject_67574625_cpu/assignment3_67574625_mux2to1.vhd" into library work

Parsing VHDL file "C:/Users/Jack/Desktop/EECS 31L/Final Project/FinalProject_67574625_cpu/assignment3_67574625_logic_unit.vhd" into library work

Parsing VHDL file "C:/Users/Jack/Desktop/EECS 31L/Final Project/FinalProject_67574625_cpu/assignment3_67574625_full_adder.vhd" into library work

Parsing VHDL file "C:/Users/Jack/Desktop/EECS 31L/Final Project/FinalProject_67574625_cpu/assignment3_67574625_arithmetic_unit.vhd" into library work

Parsing VHDL file "C:/Users/Jack/Desktop/EECS 31L/Final Project/FinalProject_67574625_cpu/pooriam_c31L_pack.vhd" into library work

Parsing VHDL file "C:/Users/Jack/Desktop/EECS 31L/Final Project/FinalProject_67574625_cpu/assignment3_67574625_alu_1bit.vhd" into library work

Parsing VHDL file "C:/Users/Jack/Desktop/EECS 31L/Final Project/FinalProject_67574625_cpu/register_bank_67574625.vhd" into library work

Parsing VHDL file "C:/Users/Jack/Desktop/EECS 31L/Final Project/FinalProject_67574625_cpu/pcounter_67574625.vhd" into library work

Parsing VHDL file "C:/Users/Jack/Desktop/EECS 31L/Final Project/FinalProject_67574625_cpu/memory_67574625.vhd" into library work

Parsing VHDL file "C:/Users/Jack/Desktop/EECS 31L/Final Project/FinalProject_67574625_cpu/controller_67574625.vhd" into library work

Parsing VHDL file "C:/Users/Jack/Desktop/EECS 31L/Final Project/FinalProject_67574625_cpu/B_input_alu.vhd" into library work

Parsing VHDL file "C:/Users/Jack/Desktop/EECS 31L/Final Project/FinalProject_67574625_cpu/A_input_67574625.vhd" into library work

Parsing VHDL file "C:/Users/Jack/Desktop/EECS 31L/Final Project/FinalProject_67574625_cpu/assignment3_67574625_alu_32bit.vhd" into library work

Parsing VHDL file "C:/Users/Jack/Desktop/EECS 31L/Final Project/FinalProject_67574625_cpu/processor_67574625.vhd" into library work

Parsing VHDL file "C:/Users/Jack/Desktop/EECS 31L/Final Project/FinalProject_67574625_cpu/testbench_processor.vhd" into library work

Starting static elaboration

Completed static elaboration

Compiling package standard

Compiling package std_logic_1164

Compiling package c31l_pack

Compiling package std_logic_arith

Compiling package std_logic_unsigned

Compiling package numeric_std

Compiling architecture behavioral of entity pcounter_67574625 [\pcounter_67574625(1)\]

Compiling architecture behavioral of entity memory_67574625 [memory_67574625_default]

Compiling architecture behavioral of entity controller_67574625 [controller_67574625_default]

Compiling architecture behavioral of entity register_bank_67574625 [register_bank_67574625_default]

Compiling architecture behavioral of entity a_input_67574625 [a_input_67574625_default]

Compiling architecture behavioral of entity b_input [b_input_default]

Compiling architecture behavioral of entity logic_unit [logic_unit_default]

Compiling architecture behavioral of entity arithmetic_unit [arithmetic_unit_default]

Compiling architecture behavioral of entity full_adder [full_adder_default]

Compiling architecture behavioral of entity mux2to1 [mux2to1_default]

Compiling architecture behavioral of entity alu_1bit [alu_1bit_default]

Compiling architecture behavioral of entity alu_32bit [alu_32bit_default]

Compiling architecture behavioral of entity processor_67574625 [processor_67574625_default]

Compiling architecture behavior of entity testbench_processor

Time Resolution for simulation is 1ps.

Waiting for 11 sub-compilation(s) to finish...

Compiled 33 VHDL Units

Built simulation executable C:/Users/Jack/Desktop/EECS 31L/Final Project/FinalProject_67574625_cpu/testbench_processor_isim_beh.exe

Fuse Memory Usage: 37900 KB

Fuse CPU Usage: 1200 ms

Launching ISim simulation engine GUI...

"C:/Users/Jack/Desktop/EECS 31L/Final Project/FinalProject_67574625_cpu/testbench_processor_isim_beh.exe" -intstyle ise -gui -tclbatch isim.cmd  -wdb "C:/Users/Jack/Desktop/EECS 31L/Final Project/FinalProject_67574625_cpu/testbench_processor_isim_beh.wdb"

ISim simulation engine GUI launched successfully

Process "Simulate Behavioral Model" completed successfully

6	Waveforms

Note: At the rising edge of clock, the output shown is saved and the memory_addrs changes

Full Waveform Snapshot 1

Snapshots of Waveform

RESET (Not a function)

NOP

MOVI (to give value to a register)

MOVI (to give value to a second register)

MOV

ADD

ADDI

SUB

SUBI

NOT

NOTI

AND

ANDI

OR

ORI

XOR

XORI

SLL

SLLI